HAZARDOUS AREA CLASSIFICATION CHECKLIST

A working checklist for HSE managers and plant engineers

Applicable standards: SANS 10108:2023  |  IEC 60079-10-1  |  IEC 60079-10-2  |  IEC 60079-14  |  IEC 60079-17

| Facility / Plant: | ____________________________ | Study Date: |
| --- | --- | --- |
| Area / Unit: | ____________________________ | Reviewer: |

SECTION 1 — PRE-STUDY PREPARATION

1.1  Core Technical Documents

| Item | Yes | No | N/A |
| --- | --- | --- | --- |
| Latest approved P&IDs available for all affected units | ☐ | ☐ | ☐ |
| General arrangement drawings / plot plans available | ☐ | ☐ | ☐ |
| Equipment layout drawings available | ☐ | ☐ | ☐ |
| Process description / operating philosophy available | ☐ | ☐ | ☐ |
| Equipment data sheets for vessels, pumps, compressors, tanks, vents, drains and package units | ☐ | ☐ | ☐ |
| Material safety data sheets / safety data sheets for all relevant substances | ☐ | ☐ | ☐ |
| Flammability data: flash point, auto-ignition temperature, LEL/UEL, vapour density, gas group | ☐ | ☐ | ☐ |
| Dust data where applicable: particle size, Kst/Pmax, layer and cloud ignition temperatures | ☐ | ☐ | ☐ |
| Normal operating pressures and temperatures confirmed | ☐ | ☐ | ☐ |
| Relief, vent, purge and drain arrangements identified | ☐ | ☐ | ☐ |
| Ventilation data for enclosed or partially enclosed areas | ☐ | ☐ | ☐ |
| Existing hazardous area drawings and previous classification reports | ☐ | ☐ | ☐ |
| Existing Ex equipment register | ☐ | ☐ | ☐ |
| Applicable site standards / engineering practices issued to the study team | ☐ | ☐ | ☐ |
| Scope boundaries for the study clearly defined | ☐ | ☐ | ☐ |

1.2  Site Access and Permit Requirements

| Item | Yes | No |
| --- | --- | --- |
| Site access approval obtained | ☐ | ☐ |
| Induction completed by all participants | ☐ | ☐ |
| Permit-to-work requirements confirmed | ☐ | ☐ |
| Escort requirements confirmed for live process areas | ☐ | ☐ |
| PPE requirements issued to all attendees | ☐ | ☐ |
| Photography / device restrictions clarified | ☐ | ☐ |
| Shutdown / live-plant limitations understood | ☐ | ☐ |
| High-risk areas identified before walkthrough | ☐ | ☐ |

1.3  Required Stakeholders

Classification shall be carried out by a qualified engineer or competent person familiar with the process, equipment, and applicable safety and electrical requirements.

| Stakeholder | Present | Not Present |
| --- | --- | --- |
| Process engineer | ☐ | ☐ |
| HSE manager / HSE representative | ☐ | ☐ |
| Electrical engineer | ☐ | ☐ |
| Instrument / control engineer | ☐ | ☐ |
| Operations representative | ☐ | ☐ |
| Maintenance representative | ☐ | ☐ |
| Project engineer / asset owner representative | ☐ | ☐ |
| Hazardous area specialist / study leader | ☐ | ☐ |

SECTION 2 — ZONE CLASSIFICATION VERIFICATION

2.1  Gas / Vapour Classification  (IEC 60079-10-1)

| Checkpoint | Yes | No | N/A |
| --- | --- | --- | --- |
| All potential release sources identified | ☐ | ☐ | ☐ |
| Release grade assessed for each source (continuous / primary / secondary) | ☐ | ☐ | ☐ |
| Process temperature versus flash point considered | ☐ | ☐ | ☐ |
| Pressurised liquid release / aerosol / mist potential considered | ☐ | ☐ | ☐ |
| Heavier-than-air gas behaviour considered for pits, trenches, drains and low points | ☐ | ☐ | ☐ |
| Lighter-than-air gas behaviour considered for roofs, canopies and elevated pockets | ☐ | ☐ | ☐ |
| Internal zones inside vessels / tanks / ducts assessed where relevant | ☐ | ☐ | ☐ |
| Outdoor and indoor release scenarios considered separately | ☐ | ☐ | ☐ |
| Impact of start-up, shutdown, sampling, filling, emptying and maintenance considered | ☐ | ☐ | ☐ |
| Zone 0: explosive gas atmosphere expected continuously, frequently or for long periods | ☐ | ☐ | ☐ |
| Zone 1: explosive gas atmosphere likely in normal operation occasionally | ☐ | ☐ | ☐ |
| Zone 2: explosive gas atmosphere not likely in normal operation; if it occurs, for a short period only | ☐ | ☐ | ☐ |

2.2  Combustible Dust Classification  (IEC 60079-10-2)

| Checkpoint | Yes | No | N/A |
| --- | --- | --- | --- |
| Dust release sources identified | ☐ | ☐ | ☐ |
| Dust cloud generation during normal operation assessed | ☐ | ☐ | ☐ |
| Dust layer formation and housekeeping assumptions recorded | ☐ | ☐ | ☐ |
| Bucket elevators, conveyors, silos, filters and bagging points assessed | ☐ | ☐ | ☐ |
| Internal dust zones assessed where applicable | ☐ | ☐ | ☐ |
| Zone 20: explosive dust cloud present continuously, frequently or for long periods | ☐ | ☐ | ☐ |
| Zone 21: explosive dust cloud likely in normal operation occasionally | ☐ | ☐ | ☐ |
| Zone 22: explosive dust cloud not likely in normal operation; if it occurs, for a short period only | ☐ | ☐ | ☐ |

2.3  Zone Boundary Determination

| Key Verification Question | Yes | No |
| --- | --- | --- |
| Is the release point physically correct on the drawing? | ☐ | ☐ |
| Has the extent been determined in three dimensions? | ☐ | ☐ |
| Are walls, bunds, pits, trenches, buildings and roofs influencing the extent? | ☐ | ☐ |
| Is the selected extent consistent with the release grade and ventilation assumptions? | ☐ | ☐ |
| Are adjacent areas, doors, windows, intakes and ignition source routes considered? | ☐ | ☐ |
| Are overlapping sources treated correctly? | ☐ | ☐ |
| Are package units and skid boundaries included? | ☐ | ☐ |
| Are temporary operating modes or abnormal-but-credible releases noted separately? | ☐ | ☐ |

2.4  Ventilation Classification

| Checkpoint | Yes | No | N/A |
| --- | --- | --- | --- |
| Ventilation identified as natural, mechanical, or both | ☐ | ☐ | ☐ |
| Basis for 'adequate ventilation' is documented | ☐ | ☐ | ☐ |
| Mechanical ventilation duty / availability / failure mode considered | ☐ | ☐ | ☐ |
| Air changes / extraction rates / dilution basis recorded where enclosed areas involved | ☐ | ☐ | ☐ |
| Ventilation dead spots identified | ☐ | ☐ | ☐ |
| Effect of blocked louvres, failed fans or closed doors considered | ☐ | ☐ | ☐ |
| Ventilation assumptions align with actual operating practice | ☐ | ☐ | ☐ |

SECTION 3 — EQUIPMENT SELECTION VERIFICATION

3.1  Equipment Protection Level (EPL) Suitability

| Checkpoint | Yes | No | N/A |
| --- | --- | --- | --- |
| Required gas EPL recorded for each tagged item (Ga / Gb / Gc) | ☐ | ☐ | ☐ |
| Required dust EPL recorded for each tagged item (Da / Db / Dc) | ☐ | ☐ | ☐ |
| Selected EPL matches the classified zone on the latest drawing | ☐ | ☐ | ☐ |
| Gas group compatibility confirmed (IIA / IIB / IIC as applicable) | ☐ | ☐ | ☐ |
| Dust group compatibility confirmed (IIIA / IIIB / IIIC as applicable) | ☐ | ☐ | ☐ |
| Temperature class / maximum surface temperature verified against substance data | ☐ | ☐ | ☐ |
| Ambient temperature range of equipment suitable for site conditions | ☐ | ☐ | ☐ |

EPL Quick Reference

| Zone | Minimum EPL Required | Zone | Minimum EPL Required |
| --- | --- | --- | --- |
| Zone 0 | Ga | Zone 20 | Da |
| Zone 1 | Gb (or Ga) | Zone 21 | Db (or Da) |
| Zone 2 | Gc / Gb / Ga | Zone 22 | Dc / Db / Da |

3.2  Protection Concept Verification

| Checkpoint | Yes | No | N/A |
| --- | --- | --- | --- |
| Protection concept identified for each item | ☐ | ☐ | ☐ |
| Ex d (flameproof) equipment applied appropriately | ☐ | ☐ | ☐ |
| Ex e (increased safety) equipment applied appropriately | ☐ | ☐ | ☐ |
| Ex ia (intrinsic safety) equipment applied appropriately | ☐ | ☐ | ☐ |
| Ex n / Ex ec / Zone 2 concept applied only where appropriate to the zone and product certification | ☐ | ☐ | ☐ |
| Ex p (pressurisation) basis and support systems reviewed | ☐ | ☐ | ☐ |
| Ex m (encapsulation) suitability reviewed | ☐ | ☐ | ☐ |
| Cable glands, stopping plugs, breathers and accessories match the protection concept | ☐ | ☐ | ☐ |

3.3  Certification Verification

| Checkpoint | Yes | No | N/A |
| --- | --- | --- | --- |
| IECEx certificate available and traceable, where claimed | ☐ | ☐ | ☐ |
| ATEX certificate / declaration available, where claimed | ☐ | ☐ | ☐ |
| Certificate matches exact model / option / temperature class installed | ☐ | ☐ | ☐ |
| Certificate has not been superseded by a limitation affecting site use | ☐ | ☐ | ☐ |
| 'X' special conditions of use identified and implemented | ☐ | ☐ | ☐ |
| 'U' component certificate not incorrectly treated as complete equipment certification | ☐ | ☐ | ☐ |
| Local site acceptance requirements for imported equipment confirmed | ☐ | ☐ | ☐ |

3.4  Nameplate Marking Verification

| Nameplate Check | Yes | No |
| --- | --- | --- |
| Manufacturer and model number legible | ☐ | ☐ |
| Ex marking legible and complete | ☐ | ☐ |
| Equipment Protection Level (EPL) shown | ☐ | ☐ |
| Gas / dust group shown | ☐ | ☐ |
| Temperature class or maximum surface temperature shown | ☐ | ☐ |
| Certificate number shown | ☐ | ☐ |
| Ambient range shown where required | ☐ | ☐ |
| IP rating shown where relevant | ☐ | ☐ |
| Serial number matches equipment register | ☐ | ☐ |

SECTION 4 — DOCUMENTATION COMPLETENESS

| Required Deliverable | Yes | No | N/A |
| --- | --- | --- | --- |
| Hazardous area classification drawings issued | ☐ | ☐ | ☐ |
| Zone boundaries clearly shown and readable | ☐ | ☐ | ☐ |
| Release points identified on drawings | ☐ | ☐ | ☐ |
| Basis of classification document completed | ☐ | ☐ | ☐ |
| Hazardous area classification report issued | ☐ | ☐ | ☐ |
| Assumptions, exclusions and limitations stated clearly | ☐ | ☐ | ☐ |
| Ex equipment register updated with certificate references | ☐ | ☐ | ☐ |
| Drawing revision numbers consistent across the package | ☐ | ☐ | ☐ |
| Review comments and close-out actions recorded | ☐ | ☐ | ☐ |
| Approval signatures complete | ☐ | ☐ | ☐ |
| Controlled document issue completed | ☐ | ☐ | ☐ |

SECTION 5 — POST-STUDY REVIEW

| Post-Study Control | Yes | No | N/A |
| --- | --- | --- | --- |
| Reclassification trigger list defined | ☐ | ☐ | ☐ |
| Trigger includes process condition changes | ☐ | ☐ | ☐ |
| Trigger includes new equipment / skids / vents / drains | ☐ | ☐ | ☐ |
| Trigger includes layout changes / buildings / enclosures / ventilation changes | ☐ | ☐ | ☐ |
| Trigger includes change in substances handled | ☐ | ☐ | ☐ |
| Trigger includes change in housekeeping assumptions for dust areas | ☐ | ☐ | ☐ |
| Trigger includes modifications raised through MOC | ☐ | ☐ | ☐ |
| Review interval defined by site procedure | ☐ | ☐ | ☐ |
| Recommended formal review interval recorded | ☐ | ☐ | ☐ |
| Findings integrated into maintenance management system / asset register | ☐ | ☐ | ☐ |
| Ex inspection programme updated to reflect revised zones | ☐ | ☐ | ☐ |

Review the classification whenever there is a material process change, layout change, ventilation change, new ignition-capable equipment, or a Management of Change event that could alter release frequency, extent or consequence.

STUDY SIGN-OFF

| Facility Name | ________________________________________ |
| --- | --- |
| Area / Unit | ________________________________________ |
| Study Date(s) | ________________________________________ |
| Study Leader | ________________________________________ |
| Report Number / Revision | ________________________________________ |
| Next Review Due | ________________________________________ |

Disclaimer: This checklist is provided for general guidance only. It does not constitute engineering advice and does not replace the applicable standards, site-specific engineering judgement, or the involvement of a qualified and registered engineer. Touch Teq Engineering accepts no liability for any loss, damage or non-compliance arising from the use of this document. Always refer to the current edition of the applicable standards.